# Audio Daughter Card Applications

## Contents

Some audio applications for the TMS320C6711 DSK with Audio Daughter Card have been provided. These include:

Alien Voices

Changing voices using ring modulation. Widely used in science- fiction films for alien voices.

### Delays and Echo

Using buffers to delay a signal up to 4 seconds to simulate echo from a valley / cavern.

### Electronic Crossover

Dividing audio signal into bass and treble using Finite Impulse Response (FIR) filters.

### Guitar Effects

Changing sound of an electric guitar by adding reverberation, treble boost and distortion.

### Guitar Tuner

Tuning the strings of an electric guitar using an adaptive filter. Display of accuracy of tuning on LEDs.

### Playback and Record

Using buffers to record and playback sounds. Half speed and double speed playback. Introduces concepts of decimation and interpolation.

### Reverberation

Simulation of reflections from walls to make a small room sound like an auditorium.

### Signal Generator

Generating test waveforms – sine and triangle, continuous, burst and sweep between 60 Hz and 12 kHz.

## Using the Applications

In order to use the applications, they must first be copied from the CD to the hard drive of the computer.

The following procedure applies to the application Alien Voices. Other projects are copied and built in a similar way. Simply rename the application.

| 1. | Connect the Audio Daughter Card to the TMS3206711 DSK, connect the cable between the DSK and the computer and power up. |
| --- | --- |
| 2. | Using Windows Explorer, go to the directory where the project is to be saved, for example Ticcs2 -> myprojects. Then select File -> New -> Folder. Highlight the new folder then right click. Rename the folder to Alien Voices. |
| 3. | Go to the directory where the files that make up the application are stored. Press control-A to highlight all the files. Right click then copy. Go back to the directory C:\ticcs2\myprojects\Alien Voices, right click to paste. |
| 4. | Important – when the files have been copied, change all the properties of each file from so that they are no longer read-only. This can be done using control-A to highlight all the files, right click then properties. Ensure that the Attributes: Read-only box is not ticked. |
| 5. | Close down Windows Explorer. |
| 6. | Start Code Composer Studio. |
| 7. | Select Project ->New. Select the location C:\ticcs2\myprojects. Set the Project Name to Alien Voices. This must be the same name used in step 2. Click on Finish. |
| 8. | Select Project -> Add Files to Project... Select the directory C:\ticcs2\myprojects\Alien Voices. Add all the Files of type C Source files (*.c, *.ccc) by pressing control then left clicking on each file to highlight them all. Click on Open. |
| 9. | A Select Project -> Add Files to Project... Select the directory C:\ticcs2\myprojects\Alien Voices. Select Files of type Asm Source Files (*.a*), highlight vectors.asm then click on Open. |
| 10. | A Select Project -> Add Files to Project... Select the directory C:\ticcs2\myprojects\Alien Voices. Select Files of type Linker Command File (*.cmd). Click on file named lnk then click Open. |
| 11. | It is not necessary to add the .h files to the project. |
| 12. | Select Project -> Rebuild All. There should be no errors or warnings. |
| 13. | Select File -> Load Program -> Debug ->Alien Voices.out then Open |
| 14. | Select Debug -> Go Main and the function main() should appear. |
| 15. | Select Debug -> Run or Press F5 to run the program. |
| 16. | The message “Audio Daughter Card. Alien Voices” should appear on the Stdout window, as well as the status of the user switches. |
| 17. | If the user switches are changed, the message on the Stdout window should also change |