# Alien Voices

## Introduction

In science-fiction films, aliens are often given strange voices. One of the favourite effects to generate these voices is known as ring modulation. This involves multiplying the voice input by a reference waveform such as a sinusoid.

## Ring Modulation

When the input cos a is multiplied by the reference cos b, sum and difference frequencies are produced.

cos a x cos b = ½ cos (A+B) + ½ cos (A - B)

As an example, say we have an input frequency of 300 Hz and are multiplying by a reference frequency of 1000 Hz. The output will contain two components: one of 700 Hz and the other of 1300 Hz. In most cases the two frequencies generated will be neither of those of the inputs and musically sound dreadful!

## C Code Implementation

The C code implementation is very simple. The input is multiplied by a reference waveform (from a table or a sweep generator) and fed to the output.

## Music Input

Ring modulation can be applied to musical signals as well as voices. The output will contain mostly wrong notes, and is therefore not widely used.

## Other Applications of Modulation

Multiplying a music or voice signal by a high frequency is the method used to transmit AM and FM radio signals. In this case, the reference frequency is referred to as the carrier.

## Additional Equipment Required

Multimedia speakers

CD player with connecting cable